# COSC 3321 – Project report

# Team members:

Member 1: Marie Garcia

Member 2: Derius Hall

Member 3: Bennie Sofiyea

Member 4: Dalton Petersen

Member 5: Avery Bowers

Member 6: Kendall Faichtinger

Member 7: Christopher Donan

Member 8: Roman Lozano

Member 9: Erik Arrona

# Moore Finite State Machine

Place here the picture of the Moore FSM.

Add notes about abbreviations used.

# Encodings

## States:

|  | S5 | S4 | S3 | S2 | S1 | S0 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 0 | 0 | 1 | 1 |
| 4 | 0 | 0 | 0 | 1 | 0 | 0 |
| 5 | 0 | 0 | 0 | 1 | 0 | 1 |
| 6 | 0 | 0 | 0 | 1 | 1 | 0 |
| 7 | 0 | 0 | 0 | 1 | 1 | 1 |
| 8 | 0 | 0 | 1 | 0 | 0 | 0 |
| 9 | 0 | 0 | 1 | 0 | 0 | 1 |
| 10 | 0 | 0 | 1 | 0 | 1 | 0 |
| 11 | 0 | 0 | 1 | 0 | 1 | 1 |
| 12 | 0 | 0 | 1 | 1 | 0 | 0 |
| 13 | 0 | 0 | 1 | 1 | 0 | 1 |
| 14 | 0 | 0 | 1 | 1 | 1 | 0 |
| 15 | 0 | 0 | 1 | 1 | 1 | 1 |
| 16 | 0 | 1 | 0 | 0 | 0 | 0 |
| 17 | 0 | 1 | 0 | 0 | 0 | 1 |
| 18 | 0 | 1 | 0 | 0 | 1 | 0 |
| 19 | 0 | 1 | 0 | 0 | 1 | 1 |
| 20 | 0 | 1 | 0 | 1 | 0 | 0 |
| 21 | 0 | 1 | 0 | 1 | 0 | 1 |
| 22 | 0 | 1 | 0 | 1 | 1 | 0 |
| 23 | 0 | 1 | 0 | 1 | 1 | 1 |
| 24 | 0 | 1 | 1 | 0 | 0 | 0 |
| 25 | 0 | 1 | 1 | 0 | 0 | 1 |
| 26 | 0 | 1 | 1 | 0 | 1 | 0 |
| 27 | 0 | 1 | 1 | 0 | 1 | 1 |
| 28 | 0 | 1 | 1 | 1 | 0 | 0 |
| 29 | 0 | 1 | 1 | 1 | 0 | 1 |
| 30 | 0 | 1 | 1 | 1 | 1 | 0 |
| 31 | 0 | 1 | 1 | 1 | 1 | 1 |
| 32 | 1 | 0 | 0 | 0 | 0 | 0 |

## State cont.:

|  | S5 | S4 | S3 | S2 | S1 | S0 |
| --- | --- | --- | --- | --- | --- | --- |
| 33 | 1 | 0 | 0 | 0 | 0 | 1 |
| 34 | 1 | 0 | 0 | 0 | 1 | 0 |
| 35 | 1 | 0 | 0 | 0 | 1 | 1 |
| 36 | 1 | 0 | 0 | 1 | 0 | 0 |
| 37 | 1 | 0 | 0 | 1 | 0 | 1 |
| 38 | 1 | 0 | 0 | 1 | 1 | 0 |
| 40 | 1 | 0 | 1 | 0 | 0 | 0 |
| 41 | 1 | 0 | 1 | 0 | 0 | 1 |
| 42 | 1 | 0 | 1 | 0 | 1 | 0 |
| 43 | 1 | 0 | 1 | 0 | 1 | 1 |
| 44 | 1 | 0 | 1 | 1 | 0 | 0 |

## Counter:

|  | C3 | C2 | C1 | C0 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 |
| 5 | 0 | 1 | 0 | 1 |
| 6 | 0 | 1 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 |

# Light:

|  | # L1 | # L0 |
| --- | --- | --- |
| # Green | # 0 | # 0 |
| # Yellow | # 0 | # 1 |
| # Red | # 1 | # 0 |

# Car:

|  | E | X |
| --- | --- | --- |
| NO | 0 | 0 |
| Yes | 1 | 1 |

# Counters

https://onedrive.live.com/edit.aspx?resid=ADA187DA6CBE3007!1938&ithint=file%2cxlsx&authkey=!AMTfej4tgkGN1Rk

# State Transitions/State To Outputs Table

https://myshsu-my.sharepoint.com/:x:/g/personal/cmd106_shsu_edu/EXwUfqMLJgdIqxN14MlVemYBTaKThpQ1WERWJPRJud0lmg?e=FEgb4S

# State Transition Boolean Equations

# State To Outputs Boolean Equations

# Implementation

Place pictures of the main components of our implementation.

# Test cases

Case 1: (Passed)

Step 1: Reset, Output: C:0, LE:G, LX:R

Step 2: Set E to 1 and X= 0 then click the clock until the count equals 8.

Step 3: Click the clock one more time. Output: C:9, LE:G, LX:R

Step 4: Click the clock once. Output: C:9, LE:G, LX:R

Step 5: Set X = 1, then the clock on when the C:9: L:Y,LX:R

Step 6: Set X and E = 0 and click the clock a few times. Output: C:9, LE:R, LX:G

Step 7: Set X = 1 and click the clock. Output: C:8, LE:R, LX:G

Case 2: (Passed)

Step 1: Reset, Output: C:0, LE:G, LX:R

Step 2: Set E= 0, and click the clock several times, the output should the remain the 	same: C:0, LE:G, LX: R

Step 3:  Set E = 1 , toggle the clock to 1, C:1, LE:G, Lx:R

Step 4: Set E = 0, X = 0, and toggle the clk several times and show the same output as 	step 3

Step 5: See X = 1 and toggle the clk twice, C:1, LE: Y, Lx:R

Step 6: Toggle the clock once, output Clk: 1, Le:R, Lx: G

Step 7: Set E= 0, X= 0, toggle the Clk several times and it should remain the same as Step 	6

Step 8: Set X = 1, Toggle the Clk: 1 , Output: C: 0, Le: R, Lx:Y

Step 9: set X= 1 Toggle the CLk: 1. Output: C:0, Le: G, Lx: R

Case 3: (Passed)

Step 1: Reset, Output: C:0, LE:G, LX:R

Step 2: Set E to 1 and X= 0 then click the clock until the count equals 8.

Step 3: Click the clock one more time. Output: C:9, LE:G, LX:R

Step 4: Click the clock once. Output: C:9, LE:G, LX:R

Step 5: Set X = 1, then the clock on when the C:9: LE:Y,LX:R

Step 6: Set X and E = 0 and click the clock a few times. Output: C:9, LE:R, LX:G

Step 7: Set X = 1 and click the clock. Output: C:8, LE:R, LX:G

Step 8: Set X= 1 and set Clk: 1. Output: C:8, LE: R, LX: G

Step 9: Set X= 0 and E=0, Toggle the Clk Several Times: C:8, LE:R, LX:G

Step 10: Set X= 0 and E= 1, Toggle the Clk: 1: C:8, LE:R, Lx:Y

Step 11: Set X = 0 and E=1, Toggle the off and on. Output: C: 8, Le:G , Lx:R

Step 12:  Set E = 0 and X=0, Toggle the CLk. Output: C:8, Le: G, Lx:R

Step 13: Set X = 1, Toggle the CLk. Output: C:8, Le: Y, Lx: R

Step 14: Set X= 1, Toggle the Clk. Output: C: 8, Le: R, Lx: G

Step 15: Set X=0 and Set E=1, Toggle the Clk; C: 8, Le: R, Lx: Y

Step  16: Set X= 0 and Set E=1, Toggle the Clk; C: 8 Le: G, Lx:Y

Step 17: Set X= 0 and  Set E=1, Toggle the Clk; C:9, Le:G, Lx: R

Case 4: PASSED

Step 1: Reset, Output: C:0, LE:G, LX:R

Step 2: Set E to 1 and X= 0 then click the clock to count 2

Step 3: Set E to 1 and X = 1 then click the clock. C:2, LE:Y, LX:R

Step 4: Toggle clock, C:2, LE:R, LX:G

Step 5: Set E to 1, X=0, toggle clock, C:2, LE:R, LX:Y

Step 6:  Toggle clock, C:2, LE:G, LX:R

Step 7: Set E=0, toggle clock, C:2, LE:G, LX:R

Step 8: Set E=1, X=0, Toggle clock, C:3, LE:G, LX:R

Step 9: Set E=1, X=1, toggle clock, C: 3, LE:Y, LX:R

Step 10: Toggle clock, C : 3, LE:R, LX:G

Step 11: Set E=0, X=0, toggle clock. C: 3, LE:R, LX:G, in S20 still

# Team member contributions:

https://myshsu-my.sharepoint.com/:x:/g/personal/kmg147_shsu_edu/EcJSMifVisROlglpR6U2AcEBkKe1updk2akwhoXb-MgGTQ?e=gcPXWt